Snugfold Specifications

Slats-Fabric hemmed around 5/8" x 2-1/2" hard maple slat at top. Three intermediate slats to be 5/16" x 1-1/4" hard maple enclosed in faced-on fabric pockets. Pockets stitched top and bottom to minimize weight on stitching. Bottom slat to be 5/16" x 1-1/4" hard maple enclosed in hem with uniform color on exterior of shade.

Stays-Three intermediate spring steel stays enclosed in taped pockets for uniform folding of shade and additional stability when shade is down.

Cord-Color coordinated cord to be #7-1/2 hard braided polypropylene.

Pulleys & Cord Locks-Shade operated by cord and pulley system. Cord locks and pulleys of 16 ga. stamped steel, brass-tone zinc plated for rust resistance, with self-lubricating nylon marine pulley wheels not less than 1-1/2" diameter. Two independent automatic cord locks for reliable operation, both housed in pulley frame, allow shade to be stopped at any height.

Valance-Matching 6" cloth valance sewn to main body of shade at top. Valance supported by continuous 5/16" x 1-1/4" maple slat resting on zinc plated "L" angles attached to top slat. Bottom of valance hemmed around a second 5/16" x 1-1/4" hard maple slat for stability and even appearance

Fabrics:

Please see the web page for available fabrics and colors.